# LC-MS/MS Method Validation Report

## Abstract

This report describes the validation of an LC-MS/MS method for the quantification of methylxanthines in biological matrices. The method demonstrates excellent linearity, precision, and accuracy within regulatory guidelines.

## 1. Introduction

Methylxanthines are a class of alkaloids that include caffeine, theobromine, and theophylline. These compounds are widely consumed and have significant pharmacological effects.

Key objectives of this validation study:

Establish linearity range (10-500 ng/mL)

Determine precision (intra-day and inter-day)

Assess accuracy at QC levels

Evaluate matrix effects

## 2. Materials and Methods

### 2.1 Chemicals and Reagents

All reference standards were purchased from Sigma-Aldrich (purity >98%). HPLC-grade solvents were obtained from Fisher Scientific.

### 2.2 Instrument Configuration

| Component | Specification | Vendor |
| --- | --- | --- |
| LC System | 1290 Infinity II | Agilent |
| MS Detector | 6495C QQQ | Agilent |
| Column | C18, 2.1×100mm, 1.8μm | Waters |
| Autosampler | Multisampler, 4°C | Agilent |

Table 1: Instrument configuration for LC-MS/MS analysis

### 2.3 Chromatographic Conditions

Mobile Phase A: 0.1% formic acid in water

Mobile Phase B: 0.1% formic acid in acetonitrile

Flow rate: 0.4 mL/min

Injection volume: 5 μL

Column temperature: 40°C

## 3. Results

### 3.1 Calibration Curve

The calibration curve demonstrated excellent linearity (R² > 0.999) over the concentration range of 10-500 ng/mL.

| Analyte | Slope | Intercept | R² |
| --- | --- | --- | --- |
| Caffeine | 1023.4 | 234.5 | 0.9998 |
| Theobromine | 892.1 | 178.3 | 0.9995 |
| Theophylline | 956.7 | 201.2 | 0.9997 |

Table 2: Calibration curve regression parameters

### 3.2 Precision and Accuracy

Intra-day and inter-day precision were evaluated at three QC levels (low, medium, high). All %CV values were below 15%, meeting FDA bioanalytical method validation criteria.

## 4. Conclusions

The developed LC-MS/MS method meets all validation criteria and is suitable for routine analysis of methylxanthines in biological samples.

## 5. References

FDA Guidance for Industry: Bioanalytical Method Validation (2018)

ICH M10: Bioanalytical Method Validation (2019)

Smith J, et al. J Chromatogr B. 2023;1234:123456